[__RemObS_________________________]             by the Jolly Roger

Some of you may have heard of devices called Remobs which stands

for Remote Observation System. These Devices allow supposedly

authorized telephone employees to dial into them from anywhere, and

then using an ordinary touch tone fone, tap into a customer's line

in a special receive only mode. [The mouthpiece circuit is

deactivated, allowing totally silent observation from any

fone in the world (Wire tapping without a court order is against

the law)]

[__How Remobs Work______________]

Dial the number of a Remob unit.  Bell is rumored to put them in the

555 information exchanges, oron special access trunks

[Unreachable except via blue box].  A tone will then be heard

for approximately 2 seconds and then silence.  You must key in

(In DTMF) a 2 to 5 digit access code while holding each digit down

at least 1 second.  If the code is not entered within 5 or 6 seconds,

the Remob will release and must be dialed again.  If the code is

supposedly another tone will be heard.  A seven digit subscriber

fone number can then be entered [The Remob can only handle certain

'exchanges' which are prewired, so usually one machine cannot

monitor an entire NPA].  The Remob will then connect to the

subscribers line.  The listener will hear the low level idle tone

as long as the monitored party is on hook.  As the monitored party

dials [rotary or DTMF], the listener would hear [And Record]

the number being dialed.  Then the ENTIRE conversation, datalink,

whatever is taking place,  all without detection.  There is no

current box which can detect Remob observation, since it is being done

with the telephone equipment that makes the connection.  When the

listener is finished monitoring of that particular customer,  he keys

the last digit of the access code to disconnects him from the

monitored line and return to the tone so that he can key in another

7 digit fone #.  When the listener is totally finished with the Remob,

he keys a single 'disconnect digit' which disconnects him from the

Remob so that the device can reset and be ready for another caller.

[_History of Remobs_______________]

Bell has kept the existance of Remobs very low key.  Only in

1974, Bell acknowledged that Remobs existed.  The device was

first made public during hearings on "Telephone Monitoring Practices

by Federal Agencies" before a subcommittee on government

operations.  House of Representatives, Ninety-Third Congress, June

1974.

It has since been stated by Bell that the Remob devices

are used exclusively for monitoring Bell employees such as operators,

information operators, etc., to keep tabs on their performance.

[Suuureee, were stupid]

[__Possible Uses for Remobs__]

The possible uses of Remobs are almost as endless as the uses

of self created fone line.  Imagine the ability to monitor bank lines

etc, just off the top of my head I can think of these applications:

Data Monitoring of:

TRW

National Credit Bureau

AT&T Cosmos

Bank Institutions

Compuserve and other Networks.

Voice Monitoring of:

Bank Institutions

Mail Order buisnesses.

Bell Telephone themselves.

Any place handling sensitive or important information.

Anyone that you may not like.

With just one Remob, someone could get hundreds of credit cards,

find out who was on vacation, get compuserve passwords by the dozens,

disconnect peoples fones, do credit checks, find out about anything

that they may want to find out about.  Im sure you brilliant

can see the value of a telephone hobbiest and a telecommunications

enthusist getting his hands on a few choice Remobs. <Grin>

[_Caution________________________]

If any reader should discover a Remob during his (or her) scanning

excursions,  please keep in mind the very strict federal laws

regarding wiretapping and unauthorized use of private Bell property.

------------------Jolly Roger

Downloaded From P-80 International Information Systems 304-744-2253